Calibrate the Touch Pannel Lab 5

For this lab you will need to run Matlab with the Tilt.m script.  You will also need the use the Tilt table but you only need USB power not the Battery power.

- Make a sheet of paper that fits over the tilt table and hold it in place with a bit of tape.

- Next mark at least 9 points in the order of:  Front Left, Mid Left, Back Left, Front Center, Center, Back Center, Right Front, Right Mid, Right Back.

- tape.  Number the dots in the same order as in 2 above.  Now use option 1 in the Tilt Table code to print out 20 samples of each point.  You can take the average in excel of each group of 20 if you want.  Just copy and paste from the terminal window to Excel.

- Next, enter the raw averages for all 9 points into the A matrix in order and be sure to include the constant 1 as a third value.  Leave the X and Y vectors as is if you have preserved the correct order.

- Then run the matlab script below:

- Then save the f1,2,3 values for A,B,C and D,E,F from g1,2,3

A =[222 341 1; 196 3566 1; 3584 3556 1;3618 354 1; 1949 2013 1;222 341 1; 196 3566 1; 3584 3556 1;3618 354 1]

x=[0; 0; 0; 1000; 1000; 1000; 2000; 2000; 2000];

y=[0; 1000; 2000; 0; 1000; 2000; 0; 1000; 2000];

f=inv(A'*A)*A'*x;

f

g=inv(A'*A)*A'*y;

g

f(3) = f(3);

g(3) = g(3);

T=[f g];

R=A*T;

R

plot(R(:,1),R(:,2),'ks',x,y,'ro')

xr = 1000;

yr = 1000;

xc = f(1)*xr +f(2)*yr + f(3);

yc = g(1)*xr +g(2)*yr + g(3);

xc

yc